Refrigeration Engineer

Pre-Employment Competency Questionnaire

| Please complete the following questions: |  | Potential Marks: |
| --- | --- | --- |
| 1 | What are the thresholds in CO2 tonnes for leak checking intervals in line with F-Gas regulations? | 1 |
| 2 | Explain a pump down cycle in a refrigeration system? | 1 |
| 3 | An A/C system is iced up give 3 reasons for this? | 3 |
| 4 | If a liquid line drier is warm at one side and has ice forming on the other explain why this is? | 1 |
| 5 | An A/C system is operating on full fresh with a DX cooling coil, what should be installed to prevent the coil from icing up? | 1 |
| 6 | If you know the air volume of an A/C system how would you determine the capacity of the cooling coil? | 1 |
| 7 | What is latent heat? | 1 |
| 8 | On air balancing a ventilation system what is meant by the system index? | 1 |
| 9 | If you know the speed of fan drive motor the fan and motor pulley sizes, how do you calculate the fan speed? | 1 |
| 10 | A supply fan is only supplying 50% of design air flow give 2 possible reasons for this? | 2 |

| 11 | Explain wet bulb temperature? | 1 |
| --- | --- | --- |
| 12 | A motor is rotating in a clock wise direction but should be rotating in a anti-clockwise direction how would you change this? | 1 |
| 13 | A refrigeration system is tripping out on high pressure give 3 reasons for this? | 3 |
| 14 | Explain a heat pump A/C system? | 1 |
| 15 | If an A/C system is tripping on low pressure give 3 reasons for this? | 3 |
| 16 | What is a VAV system? | 1 |
| 17 | Where would you use a fire damper? | 1 |
| 18 | If an AHU is producing 150% of design air flow how would you reduce this to 100%? | 1 |
| 19 | What is a VRV system? | 1 |
| 20 | If you have a power supply of 415v and you need a 24v control circuit voltage how do you achieve this? | 1 |
| 21 | If an electric heater has an inline thyristor how would you control this to maintain a constant supply air temperature? | 1 |
| 22 | What would you expect to see on an AHU inlet to remove moisture? | 1 |
| 23 | In ventilation systems what would be a normal duct velocity expected? | 1 |
| 24 | How would you size a supply fan for a ventilation system? | 1 |
| 25 | What would you use to measure air flow and how would you achieve this? | 1 |

| Total Possible Marks | Total Marks Achieved |
| --- | --- |
| 32 |  |

| Name: |  |
| --- | --- |
| Signed: |  |
| Date: |  |